COMPUTATIONAL SENSORIMOTOR CONTROL

SUPPLEMENTARY READING

Full Lagrangian Derivation for the 2-Link Planar Arm

Week 3  |  Optional Reading  |  Fall 2026

This document provides the complete step-by-step derivation of the equations of motion for the 2-link planar arm, including all intermediate algebra. The lecture notes (Week 3) present the key results; this supplement shows how every term is obtained. Notation and parameter values match the lecture notes and Gribble et al. (1998).

# A. Setup and Notation

We consider a 2-link planar arm moving in the horizontal plane (no gravity). The generalized coordinates are q = (θ₁, θ₂), where θ₁ is the shoulder angle (global frame) and θ₂ is the elbow angle (body-fixed, relative to the upper arm).

## A.1  Parameter Definitions

- l₁, l₂ — segment lengths (m)

- m₁, m₂ — segment masses (kg)

- s₁, s₂ — distances from proximal joint to center of mass (m)

- I₁, I₂ — moments of inertia about the center of mass (kg·m²)

## A.2  Numerical Values (Gribble et al. 1998)

- l₁ = 0.34 m, l₂ = 0.46 m

- m₁ = 2.10 kg, m₂ = 1.65 kg

- s₁ = 0.11 m, s₂ = 0.16 m

- I₁ = 0.015 kg·m², I₂ = 0.022 kg·m²

## A.3  The Lagrangian

The Lagrangian is L = T − V. In the horizontal plane, potential energy V = 0, so L = T (kinetic energy only). The Euler–Lagrange equation is:

d/dt (∂L/∂dθ/dtᵢ) − ∂L/∂θᵢ = τᵢ	(A.1)

# B. Kinetic Energy

## B.1  Segment 1 (Upper Arm)

The center of mass of segment 1 is at position:

p₁ = (s₁ cosθ₁,  s₁ sinθ₁)	(B.1)

Its velocity is:

dp₁/dt = (−s₁ sinθ₁ · dθ₁/dt,  s₁ cosθ₁ · dθ₁/dt)	(B.2)

The speed squared is:

v₁² = s₁² sin²θ₁ · dθ₁/dt² + s₁² cos²θ₁ · dθ₁/dt² = s₁² dθ₁/dt²	(B.3)

using sin² + cos² = 1. The kinetic energy of segment 1 (translational + rotational) is:

T₁ = ½ m₁ s₁² dθ₁/dt² + ½ I₁ dθ₁/dt² = ½ (m₁ s₁² + I₁) dθ₁/dt²	(B.4)

## B.2  Segment 2 (Forearm)

The center of mass of segment 2 is at:

x₂ = l₁ cosθ₁ + s₂ cos(θ₁ + θ₂)	(B.5a)

y₂ = l₁ sinθ₁ + s₂ sin(θ₁ + θ₂)	(B.5b)

Differentiating with respect to time. Let ϕ = θ₁ + θ₂ and dϕ/dt = dθ₁/dt + dθ₂/dt:

dx₂/dt = −l₁ sinθ₁ · dθ₁/dt − s₂ sinϕ · dϕ/dt	(B.6a)

dy₂/dt = l₁ cosθ₁ · dθ₁/dt + s₂ cosϕ · dϕ/dt	(B.6b)

The speed squared is v₂² = (dx₂/dt)² + (dy₂/dt)². Expanding:

(dx₂/dt)² = l₁² sin²θ₁ · dθ₁/dt² + s₂² sin²ϕ · dϕ/dt² + 2 l₁ s₂ sinθ₁ sinϕ · dθ₁/dt dϕ/dt	(B.7a)

(dy₂/dt)² = l₁² cos²θ₁ · dθ₁/dt² + s₂² cos²ϕ · dϕ/dt² + 2 l₁ s₂ cosθ₁ cosϕ · dθ₁/dt dϕ/dt	(B.7b)

Adding B.7a and B.7b:

v₂² = l₁² dθ₁/dt² (sin²θ₁ + cos²θ₁) + s₂² dϕ/dt² (sin²ϕ + cos²ϕ) + 2 l₁ s₂ dθ₁/dt dϕ/dt (sinθ₁ sinϕ + cosθ₁ cosϕ)	(B.8)

Key Identity: cos(α − β) = cosα cosβ + sinα sinβ The cross-term in B.8 uses sinθ₁ sinϕ + cosθ₁ cosϕ = cos(θ₁ − ϕ) = cos(θ₁ − θ₁ − θ₂) = cos(−θ₂) = cosθ₂. This is where the cosθ₂ coupling term comes from.

Applying sin² + cos² = 1 and the identity above:

v₂² = l₁² dθ₁/dt² + s₂² (dθ₁/dt + dθ₂/dt)² + 2 l₁ s₂ cosθ₂ · dθ₁/dt (dθ₁/dt + dθ₂/dt)	(B.9)

Expanding (dθ₁/dt + dθ₂/dt)² = dθ₁/dt² + 2dθ₁/dtdθ₂/dt + dθ₂/dt² and the product dθ₁/dt(dθ₁/dt + dθ₂/dt) = dθ₁/dt² + dθ₁/dtdθ₂/dt, then collecting terms:

v₂² = (l₁² + s₂² + 2l₁ s₂ cosθ₂) dθ₁/dt² + s₂² dθ₂/dt² + (2s₂² + 2l₁ s₂ cosθ₂) dθ₁/dtdθ₂/dt	(B.10)

The total kinetic energy of segment 2 (translational + rotational, noting the forearm rotates at dθ₁/dt + dθ₂/dt):

T₂ = ½ m₂ v₂² + ½ I₂ (dθ₁/dt + dθ₂/dt)²	(B.11)

## B.3  Total Kinetic Energy

T = T₁ + T₂. Collecting all terms by dθ₁/dt², dθ₂/dt², and dθ₁/dtdθ₂/dt:

T = ½ M₁₁ dθ₁/dt² + ½ M₂₂ dθ₂/dt² + M₁₂ dθ₁/dtdθ₂/dt	(B.12)

where:

M₁₁ = I₁ + I₂ + m₁ s₁² + m₂(l₁² + s₂² + 2l₁ s₂ cosθ₂)	(B.13a)

M₂₂ = I₂ + m₂ s₂²	(B.13b)

M₁₂ = M₂₁ = I₂ + m₂(s₂² + l₁ s₂ cosθ₂)	(B.13c)

Defining the inertial constants:

a₁ = I₁ + I₂ + m₁ s₁² + m₂(l₁² + s₂²) = 0.2954 kg·m²	(B.14a)

a₂ = I₂ + m₂ s₂² = 0.0642 kg·m²	(B.14b)

a₃ = m₂ l₁ s₂ = 0.0898 kg·m²	(B.14c)

The mass-inertia matrix is:

M₁₁ = a₁ + 2a₃ cosθ₂,  M₂₂ = a₂,  M₁₂ = a₂ + a₃ cosθ₂	(B.15)

# C. Euler–Lagrange Equations

We now apply the Euler–Lagrange equation (A.1) to each generalized coordinate. This is where the Coriolis and centrifugal terms emerge.

## C.1  Equation for θ₁

Step 1: Compute ∂T/∂dθ₁/dt:

∂T/∂dθ₁/dt = M₁₁ dθ₁/dt + M₁₂ dθ₂/dt	(C.1)

Step 2: Differentiate with respect to time:

d/dt(∂T/∂dθ₁/dt) = M₁₁ d²θ₁/dt² + M₁₂ d²θ₂/dt² + (dM₁₁/dt) dθ₁/dt + (dM₁₂/dt) dθ₂/dt	(C.2)

Since M₁₁ and M₁₂ depend on θ₂ only (through cosθ₂):

dM₁₁/dt = − 2a₃ sinθ₂ · dθ₂/dt	(C.3a)

dM₁₂/dt = − a₃ sinθ₂ · dθ₂/dt	(C.3b)

Substituting into C.2:

d/dt(∂T/∂dθ₁/dt) = M₁₁ d²θ₁/dt² + M₁₂ d²θ₂/dt² − 2a₃ sinθ₂ · dθ₁/dtdθ₂/dt − a₃ sinθ₂ · dθ₂/dt²	(C.4)

Step 3: Compute ∂T/∂θ₁:

T does not depend explicitly on θ₁ (it depends on θ₂ through cosθ₂, but not on θ₁). Therefore:

∂T/∂θ₁ = 0	(C.5)

Step 4: Apply Euler–Lagrange (A.1):

M₁₁ d²θ₁/dt² + M₁₂ d²θ₂/dt² − 2a₃ sinθ₂ · dθ₁/dtdθ₂/dt − a₃ sinθ₂ · dθ₂/dt² = τ₁	(C.6)

Defining h = −a₃ sinθ₂ and rearranging:

M₁₁ d²θ₁/dt² + M₁₂ d²θ₂/dt² + hdθ₂/dt² + 2hdθ₁/dtdθ₂/dt = τ₁	(C.7)

## C.2  Equation for θ₂

Step 1: Compute ∂T/∂dθ₂/dt:

∂T/∂dθ₂/dt = M₂₂ dθ₂/dt + M₁₂ dθ₁/dt	(C.8)

Step 2: Differentiate with respect to time:

d/dt(∂T/∂dθ₂/dt) = M₂₂ d²θ₂/dt² + M₂₁ d²θ₁/dt² + (dM₂₁/dt) dθ₁/dt	(C.9)

Since M₂₂ = a₂ is constant (does not depend on θ), and dM₂₁/dt = −a₃ sinθ₂ · dθ₂/dt:

d/dt(∂T/∂dθ₂/dt) = M₂₁ d²θ₁/dt² + M₂₂ d²θ₂/dt² − a₃ sinθ₂ · dθ₁/dtdθ₂/dt	(C.10)

Step 3: Compute ∂T/∂θ₂:

This is the crucial step. T depends on θ₂ through the cosθ₂ terms in M₁₁ and M₁₂:

∂T/∂θ₂ = ½ (∂M₁₁/∂θ₂) dθ₁/dt² + (∂M₁₂/∂θ₂) dθ₁/dtdθ₂/dt	(C.11)

Computing the partial derivatives:

∂M₁₁/∂θ₂ = − 2a₃ sinθ₂	(C.12a)

∂M₁₂/∂θ₂ = − a₃ sinθ₂	(C.12b)

Substituting:

∂T/∂θ₂ = − a₃ sinθ₂ dθ₁/dt² − a₃ sinθ₂ · dθ₁/dtdθ₂/dt	(C.13)

Step 4: Apply Euler–Lagrange: d/dt(∂T/∂dθ₂/dt) − ∂T/∂θ₂ = τ₂:

[M₂₁ d²θ₁/dt² + M₂₂ d²θ₂/dt² − a₃ sinθ₂ · dθ₁/dtdθ₂/dt] − [−a₃ sinθ₂ dθ₁/dt² − a₃ sinθ₂ · dθ₁/dtdθ₂/dt] = τ₂	(C.14)

Simplifying (the dθ₁/dtdθ₂/dt terms cancel):

M₂₁ d²θ₁/dt² + M₂₂ d²θ₂/dt² + a₃ sinθ₂ · dθ₁/dt² = τ₂	(C.15)

With h = −a₃ sinθ₂:

M₂₁ d²θ₁/dt² + M₂₂ d²θ₂/dt² − hdθ₁/dt² = τ₂	(C.16)

# D. Final Form of the Equations of Motion

Collecting C.7 and C.16:

M₁₁ d²θ₁/dt² + M₁₂ d²θ₂/dt² + hdθ₂/dt² + 2hdθ₁/dtdθ₂/dt = τ₁	(D.1a)

M₂₁ d²θ₁/dt² + M₂₂ d²θ₂/dt² − hdθ₁/dt² = τ₂	(D.1b)

In compact matrix form: M(q)d²q/dt² = τ − C(q, dq/dt)dq/dt, where:

M(q) = [ [a₁ + 2a₃ cosθ₂,  a₂ + a₃ cosθ₂],  [a₂ + a₃ cosθ₂,  a₂] ]	(D.2)

and C(q, dq/dt)dq/dt is the 2-vector of Coriolis/centrifugal terms:

C(q, dq/dt)dq/dt = [ hdθ₂/dt² + 2hdθ₁/dtdθ₂/dt,  −hdθ₁/dt² ]	(D.3)

with h = −a₃ sinθ₂.

## D.1  Physical Interpretation of Each Term

In the θ₁ equation (D.1a):

- M₁₁d²θ₁/dt² — shoulder acceleration times shoulder inertia (the “expected” term)

- M₁₂d²θ₂/dt² — coupling: elbow acceleration creates a torque at the shoulder

- hdθ₂/dt² — centrifugal: elbow rotation flings the forearm, creating a torque at the shoulder

- 2hdθ₁/dtdθ₂/dt — Coriolis: the interaction of both velocities creates an additional shoulder torque

In the θ₂ equation (D.1b):

- M₂₂d²θ₂/dt² — elbow acceleration times elbow inertia

- M₂₁d²θ₁/dt² — coupling: shoulder acceleration creates a torque at the elbow

- −hdθ₁/dt² — centrifugal: shoulder rotation flings the forearm outward

Why No Coriolis Term in the θ₂ Equation? Notice that D.1b has no dθ₁/dtdθ₂/dt cross-term. It cancelled exactly during the Euler–Lagrange derivation (Step 4 of C.2). This is not an approximation — it is a consequence of the Lagrangian structure. The Coriolis effect at the elbow is zero because the elbow joint is body-fixed: it only sees the centrifugal effect of the shoulder rotating, not the interaction of both velocities.

## D.2  Solving for Accelerations

Rearranging for the state derivatives used in numerical integration:

d²q/dt² = M(q)⁻¹ [ τ − C(q, dq/dt)dq/dt ]	(D.4)

Since M is 2×2, its inverse is:

M⁻¹ = (1/det(M)) [ [M₂₂, −M₁₂], [−M₂₁, M₁₁] ]	(D.5)

det(M) = M₁₁ M₂₂ − M₁₂²	(D.6)

This determinant is always positive (M is positive definite for physical inertia parameters), so the inverse always exists. Note that det(M) is smallest when the arm is fully extended (θ₂ = 0), approaching the singularity from Week 1.

## D.3  Adding Joint Damping (Non-Conservative Term)

The equations above are frictionless: the Lagrangian captures only conservative dynamics, so no damping appears. Real joints also carry viscous friction — an external, non-conservative torque that opposes joint velocity and is therefore added directly to the right-hand side rather than derived from L. Including a diagonal joint-damping matrix B (units N·m·s/rad) gives

M(q) d²q/dt² + C(q, dq/dt) dq/dt + B dq/dt = τ	(D.7)

This is Equation 4.3 of the main Week-3 notes. Throughout this derivation B = 0 (the frictionless case), but Homework 3 uses exactly this term: it compares a damped plant (Model A, B = 3.0 N·m·s/rad) against the frictionless one (Model B, B = 0), showing that direct muscle activation alone supplies too little damping for a reach to settle.

# E. Numerical Verification

As a sanity check, compute M at the reference posture θ₁ = 55°, θ₂ = 75°:

cos(75°) = 0.2588

- M₁₁ = 0.2954 + 2(0.0898)(0.2588) = 0.2954 + 0.0465 = 0.3419 kg·m²

- M₁₂ = 0.0642 + (0.0898)(0.2588) = 0.0642 + 0.0232 = 0.0874 kg·m²

- M₂₂ = 0.0642 kg·m²

- det(M) = 0.3419 × 0.0642 − 0.0874² = 0.02195 − 0.00764 = 0.01431

And h = −(0.0898) sin(75°) = −(0.0898)(0.9659) = −0.0867 kg·m².

Students can use these values to verify their DynamicsEngine implementation in Lab 03.

# References

Gribble, P. L., Ostry, D. J., Sanguineti, V., & Laboissière, R. (1998). Are complex control signals required for human arm movement? J. Neurophysiol., 79(3), 1409–1424.

Hollerbach, J. M., & Flash, T. (1982). Dynamic interactions between limb segments during planar arm movement. Biological Cybernetics, 44(1), 67–77.

Winter, D. A. (1990). Biomechanics and Motor Control of Human Movement. 2nd ed. Wiley, New York.